Path Control –

Autonomous Systems lab

מוגש על ידי:

שם פרטי: עמית      שם משפחה: דוידי            ת.ז: 208630103

Introduction

Our task is to draw a square, a Circle, and the Infinity shape with the Zumo robot.

To complete that task, we opted to use the Multi Point-to-Point path control algorithm that was depicted in class.

Implementation

1. Velocity control with PI controller

We implemented a controller in code the same way we did previously with the home-kit assignment.

Kp=1500,  Ki=400,  OutMin= -400,  OutMax=400

We also added a LPF to filter out the spikes in the odometry readings and to achieve an overall smoother result:

Filtered Velocity=α⋅CurrentVelocity+1-α⋅PreviousVelocity

α=0.5

Note – different values of α may have yielded better results, alas we did not test for that.

2. Odometry

The odometry is the mainly the same as provided, a measurement of dx is done by reading the encoderCounts and resetting it to 0, transforming the measurement to meters, and calculating dθ by dθ=dxright-dxleftWheelsDistanceInmm=0.0985

The position is then updated:

posx=currentPosX+cosθ+dθ2  ⋅dxleft+dxright2

posy=CurrentPosY+sinθ+dθ2⋅dxleft+dxright2

We tweaked the value of dθ to compensate for the imprecision of the robot’s model (single constant point of rotation) and slip.

We also calculated the velocity and passed it through the LPF mentioned above.

3. Path Control – Point-to-point

We implemented (with the help of ChatGPT) the P2P algorithm as depicted in the MATLAB simulation and the Theoretical background.

We calculate:

Vr=[cosθ     sinθ]

Vt=XDesired-posX        YDesired-posY

Where XDesired, YDesired are from the desired path (we explain in further detail on the next pages)

And posX, posY  are the position calculations from the Odometry procedure.

We then calculate the dot product Vt⋅VrVt⋅|Vr|,   and the direction value dir

We get the θt by acosVt⋅VrVt⋅Vr⋅dir

We get WForward=|Vt|

We get the trapezoid velocity profile (by limiting wforward, and the increase of wforward)

Finally, the signals that we feed into the PI Controller are:

Wforward-θt⋅0.35

Wforward+θt⋅0.35

4. Main flow – Block diagram

Fig. A block diagram of the path control program

5. Shape generation for the Paths

Infinity Shape

Math time~

To make our robot draw an infinity shape, we needed to augment the original infinity shape.

The infinity shape is given by the equations:

Γt= Xt, Yt=sint,(cost-1)⋅sin⁡(t)1+sin2t

For each point on the curve, we want a point that is 65mm vertical to it.

To achieve this, we calculate the velocity vector:

Vt=Xt, Yt=cost, cos3t-3cos2t+2sin2t+12

And for every point on the curve (t), we calculate:

up=  Normalized Vt= Xt, YtXt2+ Yt2

We then calculate the cross product Op= up  0 x 0 0 1 to get a vector perpendicular to the curve.

Finally, the point we want is given by     Γ+αOp

Fig.  Augmented Infinity shape, that a robot can follow and draw a correct infinity.

Lab results:

Measured position (from serial):

Fig . The measured position of the robot while following the augmented infinity. Serial output.

We see that the robot followed the desired path almost perfectly.

The shape drawn in lab:

Fig . the Infinity shape drawn in lab by the robot while following the augmented infinity shape.

Circle Shape

Fig . the desired Circle path fed to the robot.

Fig . the measured actual position of the robot while following the Circle path, Serial output.

The shape drawn in lab:

Fig. the shape drawn in lab by the robot, while following the Circle shape

The robot followed the path perfectly.

Letting the robot complete multiple circles without stopping it results in the following drawing:

Fig. Multiple circles drawn by the robot.

We observe a very precise and consistent movement.

Square shape

For the robot to draw a square shape, we need to augment a square, in this case we can’t use a velocity vector as we did with the infinity shape, since the edges won’t result in a smooth path for the robot to follow,

We instead make it by hand, concatenating ¼ of a circle that originates at the square’s corner, and has a radius of the pen’s offset (65mm) (Python Code is available in the google-collab notebook):

Fig . the desired square path fed to the robot.

NEED TO REDO

Fig . the measured actual position of the robot while following the Square path, Serial output.

NEED TO REDO

The shape drawn in lab:

Fig. the shape drawn in lab by the robot, while following the Circle shape

We observe that the robot follows the path almost perfectly from the odometry, however, due to slip \ imperfect point of rotation, the actual shape that was drawn is not a perfect square.

Further optimization of the mechanical model \ tweaking of θd can yield better results.

Appendix:

Google-Colab notebook that generates the desired paths:

https://colab.research.google.com/drive/1sQEZoUYBs30euKWlNJecnE6vXSFWy3KO#scrollTo=RC3hxT0_FO_Z